# Truss Placement – Technical Implementation Guide

All units are in centimeters (cm)

# 1. Step-by-Step Implementation Logic

## Step 1 – Read Input Geometry

1. Load roof data from a JSON file.
2. Extract all roof surfaces.
3. For each surface:
   - Read its polygon nodes (X, Y, Z).
   - Store nodes in the given order (do not reorder).

## Step 2 – Validate Geometry

For each surface:
- Ensure the polygon has at least 3 nodes.
- Ensure nodes form a closed polygon.
- Ensure consecutive nodes represent connected edges.
- Reject the surface if validation fails.

## Step 3 – Determine Truss Direction

- Trusses are placed perpendicular to the X-axis.
- Each truss is a vertical slice at X = constant.
- Trusses span in the Z direction (height).

## Step 4 – Calculate Truss Positions

1. Find global minimum and maximum X values from all surfaces.
2. Generate truss positions every SPACING_CM along X.
Example:
X = X_min, X_min + spacing, ..., X_max

## Step 5 – Intersect Truss with Roof Surfaces

For each truss position:
1. For each surface:
   - For each edge (node[i] → node[i+1]):
     - Check if the edge crosses the truss plane X = constant.
     - If yes, compute the intersection point.
2. Collect all intersection points for that truss.

## Step 6 – Build Truss Geometry

1. Remove duplicate intersection points (within tolerance).
2. Sort points by Z value (bottom → top).
3. Connect consecutive points to form truss members.

## Step 7 – Output Truss Data

For each truss:
- Store truss X position.
- Store ordered list of connected points.
- Optionally visualize in 2D (X–Z) or 3D (X–Y–Z).

# 2. Inputs and Outputs

## Input (JSON)

{
  "roofs": [
    {
      "surfaces": [
        {
          "id": "surface_1",
          "polygon": {
            "nodes": [
              { "point": { "x": 0, "y": 0, "z": 300 } },
              { "point": { "x": 500, "y": 0, "z": 400 } },
              { "point": { "x": 500, "y": 600, "z": 400 } },
              { "point": { "x": 0, "y": 600, "z": 300 } }
            ]
          }
        }
      ]
    }
  ]
}

Required Fields:
- x, y, z (float, cm)
- At least 3 nodes per surface
- Nodes must be ordered and connected

## Output (Internal or JSON)

{
  "trusses": [
    {
      "x_position_cm": 200,
      "points": [
        { "x": 200, "y": 0, "z": 280 },
        { "x": 200, "y": 600, "z": 360 }
      ]
    }
  ]
}

# 3. Edge Cases and How to Handle Them

## Free Edges

- If a truss intersects only one surface:
  - Discard the truss
  - Mark as invalid (no structural continuity)

## Slab / Roof Openings

- Openings are represented by missing surfaces.
- If no intersection is found at a truss position:
  - Skip that truss
  - Do not generate partial trusses

## Missing Supports

- If the lowest truss point is above a minimum support height:
  - Flag the truss as unsupported
  - Do not include it in final output

## Duplicate Intersections

- If two surfaces produce the same intersection point:
  - Merge them into one node
  - Treat as a connected joint

# 4. Acceptance Criteria

The implementation is correct if:
1. Geometry Validity
   - No truss contains fewer than 2 intersection points
   - All truss points are ordered by Z (ascending)
2. Connectivity
   - Trusses intersect at least two connected surfaces
   - No gaps exist between consecutive truss segments
3. Spacing Accuracy
   - Truss X positions differ by ±1 mm from target spacing
4. Units Consistency
   - All calculations and outputs are in centimeters
5. Edge Handling
   - Free edges and openings do not produce partial trusses
   - Invalid trusses are safely discarded

# 5. Assumptions

- Surface connectivity: Each roof surface is connected to at least one other surface via a shared edge. Isolated surfaces are out of scope.
- Connected node ordering: Connected nodes are stored consecutively in the polygon node array. The last node connects back to the first (closed polygon).
- Shared geometry consistency: Adjacent surfaces share identical node coordinates along their common edge.
- Units: All coordinates and distances are expressed in centimeters (cm).

# 6. Notes

- This guide defines a geometric truss placement PoC.
- Load paths, member sizing, and code compliance are out of scope.